EE318 Electronic Design Lab Project Report, EE Dept, IIT Bombay, April 2005

Emergency Telephone Dialer

Group No: B11

Nilesh Meshram (02007027) <nileshm@ee.iitb.ac.in>

Gaurav Barjatya (02007029) <gauravb@ee.iitb.ac.in>

Viren Diwan (02007032) <viren@ee.iitb.ac.in>

Supervisor (s) P.C.Pandey, V. K.Tandon

Abstract

An automatic emergency telephone dialer, dialing programmed numbers and relaying programmed messages at specific time or events, is developed. The design consists of three modules- Speech Processing, DTMF generator and Dialer & Telephone line interface. The messages are stored in multimedia card (MMC).UM 91214B IC is used for DTMF generation. TEA 1062 IC is used to perform speech and line interface function. The unit was tested with the help of B K Precision make Telephone Product Tester Model 1045B available in the lab. However in our unit, the Off Hook condition is not recognized and the messages are transmitted 5 times just after dialing. This can be improved and Off Hook condition checked before transmitting the message.