Case Study

Machine Parameters:

Ra=0.00327

Xq=1.5845

Xd=1.7572

X1q=1.04

X1d =0.4245

T1do =6.66

T1qo =0.44

H=3.542

D=0

F=1

IEEE type-1 Exciter data:

A=0.397

B=0.09

TR=0.01

TA=0.02

TF=1.0

TE=1.0

KA=400

KF=0.06

KE=1.0

Vrmax=6.0

Vrmin=-6.0

PSS data

Ks=10.0

Tw=12

T1=0.048

T2=0.032

T3=0

T4=0

Vsmax=6.0

Vsmin=-6.0

Hydro Governor Data

Tr=5.0

Tg=0.2

Tp=0.04

Delta=0.3

Sigma=0.05

T2=0.0

Tw=1.0

Pmax=1.2

Pmin=0.1

Disturbance used: Change in Vref from Vref0 to Vref0+0.1 at t= 1 sec

Parameter Entered through MATLAB command window:

‘Simulation time'

50

enter h and z parameters

real part of h parameter'

1

imaginary part of h parameter'

0

real part of z parameter'

0

imaginary part of z parameter'

0.25

if excitation system is included enter 1 or else 0'

1

enter type of excitation system-1for IEEE type-1,2 for static excitation system'

1

If pss is included enter1 or else 0'

1

enter 1 if turbine is included else 0'

1

1 for hydroturbine 2 for steam turbine'

2

infinite bus voltage'

1

enter 1 for PV bus 2 for PQ bus

choice'

1

enter P

Pg'

1

Vg'

1

Output Graphs: