EE609 - RADIATING SYSTEMS

Quiz - 1

Day   	: 	Friday                                                     	Total marks: 30

Date   	: 	Sep. 5, 2003 					Time: 5:00-6:00 pm

____________________________________________________________________

- Find approximate gain of an antenna in dB, whose two orthogonal measured HPBWs are 60 and 45 degrees.

- Find the diameter of a parabolic dish for 40 dB gain at 35 GHz. Assume efficiency η of the antenna is 60%.

- If the input impedance of 50 ohm co-axial fed microstrip antenna is 120 ohm at 1.575 GHz, find the reflected power in dB.

- Find the HPBW of an antenna whose E-field pattern varies as 1 + cosθ.

- Draw the current distribution of thin dipole of length i) λ  and ii) 5 λ / 2.

- Find Rin of a dipole antenna of length 10 cm at f = 10 MHz.

- For a dipole of diameter d = λ / 20, find its length so that its input impedance is real (~ 68 ohm).

- Write two methods for increasing the bandwidth of a dipole antenna. What are the expected bandwidth values for these cases?

- Write all the differences between the dipole and monopole antennas.

- A circular loop antenna has a diameter of 0.02 λ. How many turns are required to obtain Rin = 50 ohm.

- (a) For a linear array, if number of elements are increased from 4 to 16, then gain will increase by 2 / 3 / 4/ 6 dB?

(b) To avoid grating lobes, what is the maximum separation between the elements allowed for endfire array?

- Draw approximately the radiation pattern of two small vertical dipoles placed along the horizontal axis and fed with equal amplitude and phase.

- For an 8-element array with equal spacing of 0.7 λ and fed with equal amplitude and  phase, find the approximate gain and FNBW.

- Find the phase difference between the 4 elements of a linear array with spacing d = 0.6 λ for scanning the beam at an angle of 300 from the broadside. Write the values of phases of all the 4 elements.

- For a linear array, what are the advantages and disadvantages of uniform and cosine taper amplitude distribution? What will happen when cosine distribution with a pedestal is used?