EE609 - RADIATING SYSTEMS

Quiz - 1

Day   	: 	Thursday                                                     		Total marks: 30

Date   	: 	Sep. 9, 2003 						Time: 11:30-13:00 hrs

___________________________________________________________________________

- The measured HPBWs of an antenna in the two orthogonal planes are 30 and 25 degrees. Find approximate gain of the antenna in dB.

- Find the diameter of a parabolic dish antenna for 50 dB gain at 10 GHz. Assume that efficiency η of the antenna is 65%.

- For a wideband antenna, 20% reflected power is acceptable; find the corresponding values of reflection co-efficient and VSWR.

- A small dipole antenna is placed at the origin in the X-Y plane at Φ = 300. Write its radiation pattern in terms of Φ and θ.

- Find the HPBW of an antenna with an infinite ground plane, whose E-field pattern varies as 3 + 2cosθ. The angle θ is measured from broadside.

- Draw the current distribution of a thin monopole antenna of length i) λ/4  and ii) λ on infinite ground plane..

- Find Rin of a dipole antenna of length 10 cm at f = 400 MHz. The Zin of this antenna will be inductive or capacitive?

- For a monopole on a very large ground plane, find its length so that its input impedance is real at 900 MHz, if its wire diameter is taken as 2 mm.

- The diameter of a dipole antenna is increased from 5 mm to 10 mm at 100 MHz. Write the advantages and disadvantages.

- Draw a resonant slot antenna with a microstrip feed. Show clearly the length of slot, microstrip feed and its location, current and voltage distribution.

- A N-turn circular loop antenna has a diameter of 2 cm, and the wire diameter is 1 mm. It is wound on the ferrite core, whose effective permeability is 10. How many turns are required to obtain Rin = 50 ohm at 3 MHz.

- Design a linear array of isotropic antennas to obtain directivity of 12 dB in the broadside direction. Show clearly in the figure spacing, amplitude and phase values.

- Find the phase difference between the 6 elements of a linear array with spacing d = 0.7 λ for scanning the beam at an angle of 200 from the broadside. Write the values of phases of all the 6 elements.

- To avoid grating lobes, what is the maximum separation between the elements allowed if the array is required to scan up to an angle θ = 600 measured from the broadside.

- For a linear array, write all the advantages and disadvantages of uniform and non-uniform amplitude distribution.