INTERVAL QFT: A MATHEMATICAL AND COMPUTATIONAL ENHANCEMENT OF QFT

P. S. V.  NATARAJ,  S.  SHEELA, AND A. K. PRAKASH

Systems and Control Engineering, Dept. of Electrical Engineering,

# Indian Institute of Technology, Bombay, India. 400 076

# Email: nataraj@ee.iitb.ernet.in

Abstract: The paper presents an overview of a mathematical and computational enhancement of Horowitz's QFT design procedure. The enhancement uses methods of interval analysis and is called as interval QFT, or IQFT. IQFT addresses and solves some of the fundamental issues in QFT, concerning selection of design frequencies, selection of controller phases in bound generation, approximation of plant templates with finite plant sets, and generation of plant templates and controller bounds with reliability and to a prescribed accuracy. Several examples are presented to illustrate the key features of IQFT.

Keywords: Interval Analysis, Quantitative Feedback Theory, Robust Control.

1. INTRODUCTION

Over the last few decades, the quantitative feedback theory  (QFT) approach of Horowitz (1993) to robust control system design has been gaining popularity among control researchers. The QFT approach comprises of a collection of techniques for dealing with several classes of uncertain plants: linear and nonlinear, time-invariant and time-varying, lumped and distributed, single input-output and multi input-output, single-loop and multiple-loop, etc. Recently, several researchers have shown that the QFT technique is more general and powerful than other robust control approaches, see for example, (Chait and Hollot, 1990; Jayasuriya, 1993; Nwokah et al., 1992; Yaniv and Horowitz, 1987; Zhao and Jaisuriya, 1998).

The QFT approach (like all other robust control approaches) is based on mappings involving point numbers. It is the thesis of this paper that several key enhancements to the point QFT approach can be obtained by adopting a fundamentally different approach based on mappings involving intervals. The interval based QFT (IQFT) approach provides guarantees on the reliability and accuracy of the generated plant templates and controller bounds, and automatically produces error estimates. Further, the key issues of finite frequency selection in design frequency set formulation, finite plant approximation in template generation, and finite phase selection in bound generation are resolved in the IQFT approach.

The idea behind the interval analysis (IA) methods is to design algorithms, which in a single computation, do the approximation and a rigorous error analysis. With interval methods, one can directly deal with interval sets containing infinitely many points, and perform set operations such as subdivisions, unions, intersections, finding convex hulls, testing for set inclusion, testing for disjointedness of sets, etc. The basis for the systematic approach of IA methods is to combine computer arithmetic with order relations. For details of IA and its methods, the reader is referred to the book of Moore (1979).

The IA approach to QFT, or IQFT, is being pursued in the interval mathematics - QFT group at IIT Bombay. IQFT can be applied to (perhaps) any problem that can be solved with point QFT methods, but is surprisingly richer in its consequences. These are summarized below.

Interval Analysis based Template Generation (IATG) algorithms provide several guarantees: the templates are (a) guaranteed to be of prescribed accuracy, (b) guaranteed to be reliable, despite all kinds of computational errors such as round-off, truncation, and approximation, (c) guaranteed to enclose all actual template points, thereby avoiding any loss of robustness due to template approximation errors.

Interval Analysis based Bound Generation (IABG) algorithms also provide several guarantees: the bounds are (a) guaranteed to be robust against template inaccuracies, in the sense that regardless of the accuracy of the interval plant template used these can never lead to violation of the specifications, (b) guaranteed to be robust against phase discretization, so that no blip in the bounds can lead to violation of the specifications, (c) guaranteed to be reliable, despite all kinds of computational errors, such as round off, truncation, and approximation; the algorithms also offer key improvements: (d) the bounds are obtainable usually in much less time, and (e) a posteriori error estimates are readily available from the bounds.

Before proceeding to describe IQFT methods, the various ways in which the design problems can be posed in QFT are first categorized.

1.1 Classes of QFT Design Problems

Class A problem: “Suppose the design frequency is given. Then, how may the QFT designer generate the plant template and compute the controller bounds so that the latter are guaranteed to be reliable over designer-selected controller phase intervals? ”

On the other hand, at the given frequency, the QFT designer may wish to specify a priori the desired accuracy of the controller bounds, and leave it to the algorithm to automatically arrive at the appropriate controller phase intervals. This situation leads to the following problem class:

Class B problem: “Suppose the design frequency is given. Then, how may the QFT designer generate the plant template, and determine the controller phase intervals with their corresponding controller bounds, so that the bounds are guaranteed to be reliable and have a prescribed accuracy?”

At an even more advanced level, the designer may wish to specify only the desired accuracy of the controller bounds, the range of design frequencies along with the desired spacing between the bounds, and leave it to the algorithm to automatically determine whatever frequency and controller phase intervals are appropriate. This situation leads to the following problem:

Class C problem: “Suppose the design frequency range is given. Then, how may the QFT designer determine the frequency intervals, the controller phase intervals, and generate the plant templates, so that the bounds are guaranteed to be reliable, have a prescribed accuracy, and be spaced apart as desired?”

As described in the sequel, to solve problems of class A, the QFT designer can execute an IATG algorithm for generating the interval plant template and then apply an IABG algorithm to compute the controller bounds over designer-selected phase intervals (cf. section 4). To solve problems of classes B and C, the QFT designer can execute the appropriate unified procedure (cf. sections 5 and 6). The unified procedures combine template and bound generation steps for generating bounds of prescribed accuracy, and appropriate controller phase and design frequency intervals are determined automatically in the procedures. The issues of finite frequency and finite phase selection in point QFT are resolved in the unified procedures, along with the issue of finite plant approximation that troubles all point-based QFT methods of template and bound generation.

It is pertinent to note here why these  issues (or difficulties) arise in the first place in QFT. According to Moore  (1991, section 5), “Point methods and computations with ordinary floating-point numbers have no direct way of dealing with sets containing infinitely many or uncountably many points”. Further, “using point methods, there may be no indication, let alone guarantee, of the accuracy or completeness of the results”. As stated earlier, it is the thesis of this paper that the above mentioned issues in the point QFT approach can be resolved by adopting a fundamentally different approach to QFT using interval-mappings (rather than point-mappings).

The rest of this paper is organized as follows. In sections 2 and 3, an overview of the various IATG and IABG algorithms is given. In sections 4 through 6, procedures to solve design problems of class A, B, and C are outlined. In section 7, concluding remarks on IQFT are given.

All computations in this work are done on a PC / Pentium III 550 MHz machine using the interval arithmetic toolbox INTLAB of Rump  (1999) that runs under MATLAB.

2. INTERVAL TEMPLATE GENERATION

Consider a plant represented by the transfer function g(s, where n is a real vector of the plant parameters and s is the Laplace variable. Suppose the parameters ivary independently over given real intervals i so that we have a box  ={1 …n} of plant parameters.

Denote the phase angle and magnitude functions of g(s,as fang(, )= arg g(,fmag(, )= | g(,|, where  is a given frequency. Define the angle - magnitude function f as f(,) = (fang(, fmag(,). Then, the set G := {f(,),} defines a region in the angle-magnitude plane (i.e., in the Nichols chart), called the template of g(s,at the given .

Definition 2.1. The expression which arises if each occurrence of in f(,) is replaced by , if each occurrence of a pre-declared function (like sin, cos, exp, etc.) is replaced by the corresponding pre-declared interval function, and if the arithmetic operations in f(,) are replaced by the corresponding interval arithmetic operations, is called as the natural interval extension of  f(,) to . The natural interval extension of f to is denoted as F(, ).

One can compute F(,) and obtain, with a single evaluation of F, a template comprising of a single angle-magnitude rectangle. By inclusion property of natural interval extensions (Moore, 1979, Theorem 3.1) encloses the actual template G().  However, this angle-magnitude rectangle F(,) usually has a width that considerably exceeds the specified width .

Therefore, one may repeatedly subdivide (or partition) the parameter box, find the evaluations of F over the sub-boxes using interval arithmetic, and take the union of the results to get templates comprising of smaller and smaller angle-magnitude rectangles which give increasingly accurate information about the actual phase-magnitude values. It is a fundamental result in interval analysis that as the partition of the parameter box is refined, these templates will converge to the actual template. The partition or subdivision process can be stopped when the widths of all the angle-magnitude rectangles is less than .

Depending on the way the subdivision is done, IATG algorithms can be cast into three categories, as follows.

2.1. IATG using Uniform Subdivision

The uniform subdivision process has been originally suggested by Moore (1979, sec. 4.1) in a general setting of finding the range of a function, and for QFT template generation by Sardar and Nataraj (1997), as follows.

First, find a uniform subdivision factor N for all parameter intervals, making use of an inequality relation in (Moore, 1979, equation 4.5). Then, subdivide each parameter interval into N equal subintervals with this subdivision factor, and create a so-called uniform subdivision partition. Lastly, evaluate F(, ) over the sub-boxes of the uniform partition, in a parallel manner, using vectorized operations. It can be shown from the work of Moore (1979, sec. 4.1) that every resulting angle-magnitude rectangle is of width at most , and that the collection of all these rectangles constitutes the required template, which necessarily contains the actual one.

The advantage of this algorithm is that it is a single step algorithm – the template can be generated with a single interval evaluation of F for each sub-box of the partition, and moreover, this can be done in a parallel manner for all the sub-boxes using vectorized interval arithmetic operations. The disadvantage of this algorithm is that N is usually heavily overestimated, due to overestimation in the Lipschitz constant calculation (Rall, 1981) and in Moore's inequality referred above. Consequently, a much larger number of sub-boxes than required will be generated with such a N. Therefore, it is clear that the amount of computational effort may be rather large in this algorithm, which may lead in turn to correspondingly large computation times.

2.2. IATG  using Adaptive Subdivision

The adaptive subdivision process is also well-known in general setting of range finding in IA literature, see, for example, Kearfott (1987). Nataraj and Sardar (2000b) have also recently used it, together with the interval Gauss-Seidel method, for template generation as follows.

First, evaluate F over the current parameter box, and check the width of the resulting angle-magnitude rectangle against the specified maximum width. If the specified width is exceeded, then bisect the box into two sub-boxes by cutting in the coordinate direction in which the box is longest. Discard the original box. Pick any one of the two sub-boxes, and put the other sub-box in a processing list. Successively subdivide the picked (current) sub-box till the width of the resulting angle-magnitude rectangle is at most . Then, write the angle-magnitude information to a solution list, and discard the current sub-box. Repeat the above for all sub-boxes created in this process. Finally, output the generated template as the collection of all angle-magnitude rectangles present in the solution list.

The advantage of this algorithm is that it generates a considerably much smaller number of angle-magnitude rectangles in the template, because a sub-box is successively subdivided into smaller sub-boxes only as long as the angle-magnitude rectangle width is unacceptable. This can therefore be viewed as an adaptive subdivision process. However, each sub-box is processed sequentially. Therefore, the algorithm generally takes considerably more time than the above one involving parallel computations.

2.3. IATG using Parallel-Adaptive Subdivision

The attractive feature of the uniform subdivision based approach is parallel evaluation of F over all sub-boxes of a partition; while that of the adaptive subdivision based approach is adaptive subdivision. An algorithm that combines these two advantageous features i.e., parallel functional evaluation and adaptive subdivision is the so-called parallel-adaptive (PA) algorithm by Nataraj and Sheela (2001b).  The PA algorithm for template generation runs as follows.

In the first or initial iteration, create a partition by subdividing all parameter intervals into subintervals of nearly equal width. Then, perform parallel evaluation of F over the sub-boxes of the partition, and save all angle-magnitude rectangles, whose widths are less than  in a solution list, discarding the corresponding sub-boxes from any further consideration. In the second and succeeding iterations (only those sub-boxes for which the corresponding angle-magnitude rectangles have unacceptable widths are left), cut simultaneously all sub-boxes in the longest direction, discard the original sub-boxes used for cutting, and perform parallel evaluation of F over the sub-boxes resulting from cutting. Save all angle-magnitude rectangles whose widths are less than  in the solution list, discarding the corresponding parameter sub-boxes from any further consideration. Terminate the iterations when there are no more sub-boxes to deal with.

The advantage of the PA algorithm over the uniform subdivision algorithm is that the former usually generates a much smaller number of angle-magnitude rectangles. This is due to the usage of adaptive subdivision process in the former algorithm, which keeps the amount of computations to reasonable limits, and in turn results in significant reductions in computation times. In contrast, a much large number of sub-boxes are typically created in the uniform subdivision partition, due to heavy overestimation in the subdivision factor. This thereby leads to large execution times. The advantage of the PA algorithm over the adaptive subdivision algorithm is that the former usually executes much faster since all sub-boxes under consideration are cut in parallel (i.e., simultaneously), F is evaluated in parallel over all the resulting sub-boxes. Recall that in the adaptive subdivision algorithm, the sub-boxes are adaptively cut and processed sequentially, which slows down the processing due to its sequential nature.

In several examples, the PA algorithm is found to be faster than the above two categories by 1-2 orders of magnitude, see Nataraj and Sheela (2001b). The efficiency of the PA algorithm, in terms of the final number of template rectangles generated and the algorithm execution time, is greatly influenced by the selection of the co-ordinate direction along which boxes are subdivided in each iteration. In the version of the PA algorithm just described, the rule of subdivision is to cut each box along its longest direction. However, in many examples it is observed that this subdivision rule merely increases the number of template rectangles without yielding any significant reduction in their widths. Therefore, to further improve upon the efficiency of the PA algorithm, more efficient subdivision rules need to be explored. These are next described.

2.4. IATG using Parallel-Adaptive with Other Subdivision Rules

In the so-called parallel-adaptive with back-tracking algorithm proposed by Sheela and Nataraj (2001b), the subdivision rule is to adaptively cut the boxes along a favorable direction, i.e., in a direction along which the width of F over the sub-boxes of partition is reduced at least by some acceptable percentage.

In the so-called parallel-adaptive with first-box algorithm proposed by Nataraj and Prakash (2001), the subdivision rule is as follows. In a given iteration, pick the first box from the list of parameter boxes to be processed and bisect it along the first co-ordinate direction to get two sub-boxes. Then, evaluate F over these sub-boxes, and find the maximum width of F. Repeat the process in all other co-ordinate directions, and determine the co-ordinate direction in which the corresponding maximum width of F is the least. This co-ordinate direction is then used as the direction for subdividing all boxes in the current iteration. The parallel-adaptive algorithm can also be based on other subdivision rules found in the literature on interval branch and bound algorithms for global optimization, see, for instance, Ratz and  Csendes (1995).

2.5. Illustrative Examples

The PA algorithm with the various subdivision rules is applied to generate the interval plant templates on a suite of eleven transfer function examples listed in the Appendix. The templates of all transfer functions are generated to an accuracy of 1 deg. and 1 dB. The findings of this study are summarized in Table 1. In this Table, Rule A is the rule of subdivision along the longest co-ordinate direction used in the basic version of the PA algorithm. Rule B is the maximum smear rule in Ratz and Csendes (1995), and commonly used for global optimization. The number of uncertain parameters is given as n. A “-” in the Table indicates that the computer runs out of memory.

The performances with the various subdivision rules are compared, in terms of solution boxes and the execution time in seconds taken to generate these boxes. Summarizing the first-box subdivision rule is seen to be the best choice overall, in terms of consistency, number of template rectangles, and time taken

Table 1. Performances of Parallel-adaptive IATG and boundary extraction algorithms

|  | Example |  | n | Soln |  | Rule A |  | Rule B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Under damped |  |  | 2 | boxes time(s) |  | 30,223 7.24 |  | 281,814 109 |  |
| 2 | DC Motor |  | 2 | boxes time(s) |  | - - |  | 3,048 4.11 |  |
| 3 | Simple poles |  | 3 | boxes time(s) |  | 7,712 3.27 |  | 627 1.88 |  |
| 4 | Non-minimum phase |  | 3 | boxes time(s) |  | 512 1.33 |  | 808 1.59 |  |
| 5 | Non-rational |  | 3 | boxes time(s) |  | 6,759 7.69 |  | 35,404 35.23 |  |
| 6 | Electro- Mechanical |  | 3 | boxes time(s) |  | - - |  | - - |  |
| 7 | Vehicle clutch |  | 3 | boxes time(s) |  | 30,424 6.09 |  | 3,109 2.15 |  |
| 8 | Multiple lags |  | 4 | boxes time(s) |  | 235,139 1.23e3 |  | 411,118 1.27e3 |  |
| 9 | Mechanical |  | 5 | boxes time(s) |  | 17,320 10.89 |  | - - |  |
| 10 | Aircraft |  | 5 | boxes time(s) |  | 40,002 10.73 |  | 966 1.92 |  |
| 11 | Inv. pendulum |  | 7 | boxes time(s) |  | - - |  | 208 1.312 |  |
| Table 1 (Contd) |  |  |  |  |  |  |  |  |  |
| Ex. | Soln | Back-tracking |  |  | First-box |  | Boundary extraction |  |  |
| 1 | boxes time(s) | 25,526 3.5 |  |  | 32,201 4.86 |  | 1,053 16 |  |  |
| 2 | boxes time(s) | 2,451 1.5 |  |  | 2,860 1.11 |  | 285 0.4 |  |  |
| 3 | boxes time(s) | 459 0.74 |  |  | 627 0.46 |  | 128 0.08 |  |  |
| 4 | boxes time(s) | 504 0.3 |  |  | 512 0.4 |  | 138 0.08 |  |  |
| 5 | boxes time(s) | 6,759 3 |  |  | 6,891 3.55 |  | 436 2.9 |  |  |
| 6 | boxes time(s) | 84,466 16 |  |  | 92,608 21.99 |  | 1,604 84 |  |  |
| 7 | boxes time(s) | 2,644 0.79 |  |  | 2,844 1.81 |  | 319 0.6 |  |  |
| 8 | boxes time(s) | 170,448 633 |  |  | 189,903 103 |  | 2369 70 |  |  |
| 9 | boxes time(s) | 1,751 1 |  |  | 1,981 0.99 |  | 579 0.8 |  |  |
| 10 | boxes time(s) | 642 1 |  |  | 1,508 1.06 |  | 131 0.1 |  |  |
| 11 | boxes time(s) | 194 0.7 |  |  | 208 0.64 |  | 101 0.05 |  |  |

2.6. Properties of IATG Algorithms

Mathematical reliability of the IATG algorithms readily follows from the existence of well-known theorems in interval analysis and usage of exact interval arithmetic in all computations. For instance, it follows immediately from the inclusion property of natural interval extensions (Moore, 1979, Theorem 3.1) that, for any  > 0, the generated templates indeed enclose the exact templates. Moreover, it is a fundamental result in interval analysis (Moore, 1979, Theorem 4.1) that as the partitions are refined, the enclosures will converge to the actual ranges of the values over the given domain sets. From this property, it follows that in the limiting case of the specified width equal to zero, IATG algorithms yield templates that converge to the exact templates. Further, for any finite  > 0, IATG algorithms generate the required templates in a finite number of steps. This can be shown by proceeding on similar lines to Kearfott  (1987, Theorem 2.10). Lastly, rigorous justification of the subdivision processes in a general setting are given by Moore (1979) and Kearfott (1987),

Computational reliability of the IATG algorithms means that the algorithms are stable when  implemented on floating-point systems. The IATG algorithms can be made computationally reliable by implementing them in any interval arithmetic compiler. An interval arithmetic compiler uses machine interval arithmetic (MIA) in all computations, see, for instance,  (Klatte et al.,1993 ).

Efficiency of implementation. IATG algorithms are efficient to implement as they are simple from a computer programmer's point of view, and as their performances are not sensitive to details of implementation or to any “tuning”.

Range of application of IATG algorithms is very vast, as the algorithms are applicable to any programmable transfer function that is continuous in its parameters.

2.7. Boundary Extraction

Similar to the case of the point template, it can be shown that for a simply connected interval plant template only the boundary template rectangles need to be considered in designs. Now, the set of boundary rectangles of an interval plant template is obtainable as the union of upper, lower, left and right boundary rectangles in the Nichols chart (by upper boundary rectangles we mean the template rectangles with the maximum magnitude at each phase, and so on).

Nataraj and Sheela (2001a) present an algorithm to extract all the boundary rectangles from a given interval template, without introducing any kind of boundary approximations. The procedure given therein to extract upper boundary rectangles from an interval template is as follows: Start always by marking the rectangle forming the left end of the template as a boundary rectangle, and set it as current rectangle. Then, journey along the top edge of the current rectangle towards the right end, checking if any upward jump to a higher magnitude occurs on this journey.  IF yes, mark the rectangle associated with the jump as a boundary rectangle, set it as current rectangle, and take the jump along the left side of the current rectangle to reach its top edge. Next, continue the journey along the top edge of the current rectangle towards the right end, and so on, as before. IF no, complete the journey to reach the right end of the top edge, and when the right end of the entire template itself is thereby reached, print out all the boundary rectangles and exit the procedure; otherwise, journey downwards along the right side of current rectangle to the next lower magnitude rectangle, mark the latter rectangle as a boundary rectangle, set it as current rectangle, and journey along the top edge of the current rectangle towards the right end, and so on, as before.

The procedure to extract lower boundary rectangles is the same as to that for upper boundary rectangles, except for a few obvious modifications, such as journeying along the bottom (instead of top) edge of each current rectangle, and checking for any downward (instead of upward) jumps to a lower (instead of upper) magnitude on the journey. The procedure to extract left and right boundary rectangles is derived by simply interchanging the roles of magnitude and phase in the above procedures. Finally, the set of all boundary rectangles of the interval template is obtained by taking the union of the upper, lower, left and right boundary rectangles.

The results of boundary extraction obtained on the suite of eleven transfer function examples referred to earlier, are given in Table 1 in the last column. The Table shows that the proposed algorithm extracts the boundary rectangles quite efficiently in terms of computational time. Typically, the algorithm takes about 10-40 milliseconds to extract the boundary rectangles from a set of 100 interval template rectangles - indicating the efficiency of the boundary extraction algorithm.

3. INTERVAL BOUND GENERATION

In this section, IABG algorithms for the various specifications are described. The robust sensitivity reduction case is first taken up.

3.1  Robust sensitivity reduction

Consider the plant family {g(s,}embedded into a single-loop system with controller (s).Then, the robust sensitivity reduction specification (spec) at a given frequency  is

1+(j). g(j,-1     ws(

The quadratic constraints algorithm in (Chait and Yaniv, 1993) computes the bounds at as follows. Substituting the polar forms =kej and g=gej in the above spec and simplifying gives the quadratic inequality (dropping the argument j),

g2 k2 + 2 g k cos(x) +zs 0

where zs :=1-1/ ws2, x :=  + . The quantity x is the phase of the loop transmission function. Setting the LHS of above inequality to zero and solving for the roots gives

(1)

krootu,krootl:=−cosx+cos2x−zsg

Thus, for the plant g, the allowable magnitude range of the controller for satisfying the sensitivity reduction spec at   is [0, klroot]  [kuroot, ). Then, for the entire template G the allowable magnitude range of the controller at  [0, klbound]  [kubound, ), where kubound = max{kuroot,};  klbound= min{klroot}. The quantities kubound , klbound are called as the upper and lower bounds on the controller magnitude at . The procedure can be repeated for all  [ -2,0] to obtain the bounds over the entire controller phase range.

The quadratic constraints approach generates bounds at point values of controller phase and frequencies, and is based on point plant templates. The quadratic constraints approach can be extended for generating bounds at interval values of controller phase, and even for interval frequencies, using interval templates, as described below.

It is assumed that in the sequel that all interval templates are closed and simply connected.

3.1.1. Computing the root intervals. Consider a template rectangle picked from the interval plant template at a given frequency , and denote its magnitude interval as G and phase interval as . Let the controller phase be also an interval . Then, substituting all these interval quantities in the RHS of (1) gives

(2)  Krootu:=Q+G:=−cosX+cos2X−zsG

Krootl:=Q−G:=−cosX−cos2X−zsG

where ,

(3)

X= + 

By analogy with the point case, X is the phase interval of the loop transmission function, and Krootu, Krootlare the upper and lower root intervals at phase interval X for the considered template rectangle.

Computation of these root intervals directly from (2) using interval arithmetic produces overestimation, due to multiple occurrences of the intervals  and  in the functional expressions and the interval dependency effect. However, if the computations are done as per Theorem 2.2 in Nataraj and Sardar (2000a), then the exact values of the root intervals are obtained.

3.1.2. Computing the working phase interval. It is shown by Nataraj and Sardar (2000a)  that any bound computations for this spec need to be performed only for those X belonging to the so-called working phase interval Y defined as

(4)

Y:=−(arccos(−zs),arccos(zs)+π)

Elsewhere in the range [-2,0], any controller magnitude is satisfactory to achieve the spec, so there is no need to compute the bounds at such phases.

3.1.3. Composing the bound intervals.  Consider a template rectangle in the interval template. Using (2), the root intervals are computed at various X in the working phase range Y.  From these X and using the phase interval  of template rectangle, the corresponding controller phase intervals Θare back-calculated from (3) and the root intervals at X are reassigned to the respective . The obtained results are plotted in the Nichols chart as 2-dim interval vectors or rectangles given by Θ,Krootu and  Θ,Krootl, with the controller phase intervals   on the x-axis and the root intervals on the y-axis.

The procedure is repeated for all template rectangles, to obtain a Nichols plot of the upper and lower root intervals versus controller phase intervals for the entire interval template.

At each , the uppermost and lowermost root intervals are picked from this plot and designated as the upper and lower bound intervals. The remaining root intervals are discarded.

3.1.4. Composing bounds from bound intervals. At each  the upper and lower bounds are obtained as the maximum and minimum of the upper and lower bound intervals. Now, for any interval, the maximum and minimum occur at the right and left endpoints of the interval. Hence, the upper and lower bounds are simply the left and right endpoints of the respective bound intervals at each .

3.1.5. Computational reliability.  All computations described in this section (as elsewhere in this paper) are done using machine interval arithmetic, which automatically takes care of all kinds of computational errors, such as round off, truncation, and approximation. This produces bound values that are not only exact for the given interval plant template, but are also reliable.

3.1.6. IABG algorithm for robust sensitivity reduction specification.  The main steps of the IABG algorithm in Nataraj and Sardar (2000a), called as Algorithm SRI, can now be outlined.

First, any of the IATG algorithms described in section  2 is used to generate the interval template at the given frequency, and the set of boundary template rectangles extracted from the template using the boundary extraction algorithm of Nataraj and Sheela (2001a).

Then, the working phase interval Y is found as per section 3.1.2, and subdivided into several phase subintervals of say, 5deg. phase widths.

Next, the root intervals are computed as per section 3.1.1 at each of these subintervals, and the bound intervals composed as per section 3.1.3.

Lastly, the bounds are composed from these bound intervals as per section 3.1.4.

3.2. Robust gain-phase margin specification

At a given frequency , the robust gain-phase margin specification can be written as

(j).g(j,/  (1+(j).g(j,  wm(

The IABG algorithm for this spec, called as Algorithm RSBI in Nataraj and Sardar (2000a),  is identical to the one in the preceding subsection, except for very some minor changes in the RHS expressions in (2).

3.3. Robust tracking specification

At a given frequency , the robust tracking specification can be written as

(5)

(j). gi(j,/ ( 1+(j). gi(j,                                                           

(j). gk (j,/ ( 1+(j). gk(j,

for all pairs of plant elements gi , gk in the plant family. Consider an arbitrary but fixed pair of boundary rectangles and recall that G denotes the magnitude interval and Φ the phase interval of a template rectangle. Then, by substituting these quantities in (5) squaring both sides, and simplifying gives the quadratic inequality

Gi2Gk21−1δ2k2+2Gk2Gicos(Φi+Θ)−GkGi2δ2cos(Φk+Θ)k+Gk2−Gi2δ2≥0

Making the substitutions X1 =1/Gi, X2=1/Gj, X3 = i + , X4 = k + and solving the above with equality sign gives

(6)

Kuroot, Klroot  = ( (X2 CosX4 / 2 - X1 Cos X3 )  Sqrt ((X2 CosX4 / 2 - X1 Cos X3 )2 - (1-1/2) (X12 - X22 / 2 ) )  ) / (1-1/2)

Thus, for the considered pair of boundary template rectangles, the allowable magnitude range of the controller at  is [0, min Krootl][maxKrootu, ). The allowable magnitude range of the controller at  for the entire interval plant template is obtained by taking the intersection of the magnitude ranges over all pairs of boundary template rectangles.

Just as is the case with (2), direct interval arithmetic evaluation of (6) produces overestimated values of Krootuand Krootl, due to multiple occurrences of the intervals X1, ..., X4  in the functional expressions and the interval dependency effect. However, by combining the concept of monotonicity with the tool of subdivision, the root intervals can be obtained directly and exactly, with no need for any initial guess values or algorithmic iterations (as in classical optimization methods). Then, the computed bounds also turn out to be exact for the given interval template. These ideas form the basis of the IABG algorithm in Nataraj (2001), called as Algorithm TSI that generates tracking bounds.

Figure 1.Inner (upper solid line) and outer (lower solid line) tracking bounds generated by IABG Algorithm TSI for the aircraft example at w=0.1. The stars denote the tracking bounds generated using the QFT toolbox. The QFT design problem is of class A.

The procedures to solve various classes of QFT design problems are next given.

4. PROCEDURE FOR CLASS A PROBLEMS

At a given design frequency, any of the IATG algorithms mentioned in section 2 can be applied to generate the interval plant template, reliably and to a prescribed accuracy. The respective IABG algorithm in section 3 can then be applied to this interval plant template to compute the controller bounds over designer-selected phase intervals. This approach provides a solution to QFT design problems of class A.

4.1. Illustrative Example

Algorithm TSI mentioned in section 3.3 is illustrated on an aircraft example. At a given frequency , the interval template and the tracking bounds are generated to satisfy the robust tracking specification.

Example 4.1.  The transfer function for the longitudinal motion of an aircraft in open-loop is given as Example 10 in the Appendix. The nominal values are as follows: k0=2, z0=0.5, p0=10, n0=6, 0=0.8. The tracking bounds for the above plant at =0.1 with the tracking specification as =1.03 are required. The controller phase intervals are to span the range [-2, 0] with a width of 5 deg.

Notes: The design frequency is selected as =0.1. The controller phases at which bounds are to be generated are also selected (spaced 5deg. apart) as is usually done in point QFT methods. The accuracy of

the bounds is not prescribed (only reliable bound values are wanted by the designer). This puts the design problem in class A.

Solution: The various steps of the solution are as follows. The interval template is generated at =0.1 using the IATG algorithm of Sheela and Nataraj (2001b). This gives an interval plant template comprising of 70 rectangles, and takes 0.82 seconds and 85,480 flops. From the interval plant template, the set of boundary template rectangles is extracted using the algorithm in Nataraj and Sheela (2001a). This gives a set of 47 boundary template rectangles, and takes just 0.02 seconds and 88 flops. Algorithm TSI is applied to this set of boundary rectangles to generate the tracking bounds on the nominal loop transmission function L0(0.1j)=(0.1j) g0(0.1j), as the plots on L0 are usually preferred to those on (.) in QFT designs. Algorithm TSI takes 7.58 seconds and 5,817,592 flops and the resulting plot of tracking bounds on L0(0.1j) is shown in Figure 1 as the upper solid line.

For comparison, the algorithm available in the QFT toolbox is also applied. Each uncertain parameter is rastered at the minimum, mean and maximum values, as is the common practice in QFT. With such a rastering, the QFT toolbox gives a template consisting of 35=243 plants. The bound generation algorithm in the QFT toolbox takes 234 seconds and 5,817,592 flops. Thus, the IABG algorithm TSI requires about 46 times less flops than the QFT toolbox algorithm, and executes about 31 times faster than the latter.

Comments on the error estimates: The lower solid line in Figure 1 is the so-called outer estimate of the tracking bounds. Using the outer and inner estimates, the maximum possible error at any particular phase is obtained as the difference between the corresponding inner and outer values.

The maximum possible error in the generated tracking bounds is the maximum of these differences over the entire phase range. From Figure 1, this is seen to be about 1.5 dB.

5. PROCEDURE FOR CLASS B PROBLEMS

IATG and IABG algorithms can be combined to form a unified procedure that solves QFT design problems of class B. The unified procedure in Nataraj and Sheela (2001c) produces controller bounds of prescribed accuracy, for the robust sensitivity reduction and robust gain-phase margin specification problems. Moreover, the proposed procedure also completely solves the problem of phase selection inherent in point QFT procedures.

To solve problems in this class, firstly an acceptable size of template rectangles needs to be determined and specified to the IATG algorithms, as follows.

5.1. Size of Template Rectangles for Class B

Let  denote the desired accuracy of the bounds in dB, and let  be split into two positive quantities Q and G such that their sum equals . Further, let the working phase interval Y be subdivided such that width of Q  in (2) is at most Q on every subdivision. Let w denote the smallest width of these subdivisions.

Then, it is shown by Nataraj and Sheela (2001c) that an acceptable size of the interval plant template (for specifying to the IATG algorithms) is as follows: the magnitude and phase sides of each template rectangle have lengths at most G and  w, respectively.

5.2. The Unified Procedure

The steps of the unified procedure for robust sensitivity reduction specification are

- Split the accuracy: The accuracy  is split into two positive quantities Q and G such that their sum equals .

- Subdivide working phase interval: The working phase interval Y in (4) is successively bisected till over every subdivision of Y, the width (in dB) of function Q in (2) is at most Q, where X denotes a subdivision. The smallest width of these subdivisions is denoted as w.

- IATG Step: Using any IATG algorithm mentioned in section 2 an interval plant template comprising of template rectangles of prescribed size is generated: each template rectangle should have its magnitude side of length at most G dB, and phase side of length at most w. Then, the boundary template rectangles are extracted, while the rest are discarded.

- IABG Step : Using the X intervals obtained above, the bound intervals are composed as in section 3.1.3, and then the bounds are composed from these bound intervals as in section 3.1.4.

The procedure is identical for the robust gain-phase margin specification, except that the respective algorithm is used in the IABG step above.

5.3. Illustrative Example for Class B Problems

The unified procedure is illustrated on a mechanical system, for generation of robust gain-phase margin bounds to a prescribed accuracy.

Example 5.1. The transfer function of a mechanical system is given as Example 9 in the Appendix. The design frequency is =8, and the gain-phase margin spec is wm=2.3 dB. The prescribed accuracy on the robust gain-phase margin bounds is  =2.57 dB.

Notes: The design frequency is selected as =8. The controller phases at which bounds are to be generated, however, are not selected, but are rather left to be appropriately determined by the procedure. The accuracy of the bounds is, however, prescribed. This puts the design problem in class B.

Solution: The unified procedure is used to generate the gain-phase margin bounds of prescribed accuracy. Some details of the procedure are given below. The prescribed accuracy =2.57 dB is split as Q =1.57, and G =1 dB, so that =Q + G. The working phase interval is successively bisected into smaller intervals till the width of Q functions over each of these intervals does not exceed Q =1.57. The subdivision process takes about 0.3 seconds, and 15 subintervals are generated. The minimum width of the intervals is found as w=0.0137rads. Next, the interval plant template is generated using the parallel-adaptive with back-tracking algorithm given by Sheela and Nataraj (2001b). The acceptable size of each template rectangle is specified to this algorithm: each template rectangle should have its magnitude side of length at most equal G =1 dB, and its phase side of length at most equal to w=0.0137 rads. The interval template is generated in about 1 second, as a collection of 1751 template rectangles. The boundary extraction algorithm of Nataraj and Sheela (2001a) extracts 579 boundary template rectangles, and takes about 0.8 seconds. IABG algorithm RSBI in Nataraj and Sardar (2000a) is executed to generate the controller bounds. The time required for executing this step is about 3 seconds, and the total flops is 50,675. The results are plotted in Figure 2. (For comparison, the results of the QFT toolbox using a phase grid spaced 5deg apart are also plotted in the same figure, with marking `*').

.

Figure 2. Robust gain-phase margin bounds for the mechanical system obtained using the Unified procedure. The specified bound accuracy is 2.57 dB, which is almost exactly achieved. Note that the phase intervals are of varying widths. The QFT design problem is of  class B, with the frequency given as =0.8.

Comments on the accuracy of the bounds: The maximum possible error in the generated bounds is readily obtainable from the widths of the plotted intervals in Figure 2. From Theorem A.1 in Nataraj and Sheela (2001c), the error in the generated bounds is no greater than the maximum width (measured along the y-axis) of the bound intervals. This maximum width is found to be 2.564 dB, and occurs for the upper bound interval [70.2686,72.8320] over the phase interval [ -323.6288,-323.5905] deg, and for lower bound interval [56.6548,59.2187] over the phase interval [-221.5007,-221.4489] deg. Hence, the maximum error in the generated bounds does not exceed 2.564 dB. This maximum error is less than the prescribed accuracy of =2.57 dB, confirming that the generated bounds are indeed of prescribed accuracy.

Comments on the automatic selection of phase intervals: The controller phase intervals plotted in Figure 2 are found to have varying widths, with the maximum width as 50 deg and the minimum width as 0.0002deg. The widths vary because the phase intervals are adaptively and automatically found in the unified procedure (in the subdivision step), so that bounds of prescribed accuracy can be generated. In contrast, in the point QFT procedures, the designer arbitrarily selects the controller phase points at which the bound values are to be generated. Thus, the unified procedure completely solves the problem associated with phase selection in the point QFT procedures.

6. PROCEDURE FOR CLASS C PROBLEMS

The developments given above for the point frequency case can be extended to the interval frequency case. Extensions of some key concepts are first outlined, followed by the procedure to solve class C problems.

Let 0 denote the given design frequency range, and  denote a fixed but arbitrary frequency interval in 0.

Natural interval extensions. The natural interval extension in Definition 2.1 is extended for the interval frequency case as follows: The expression which arises if each occurrence of  and  in fmag (, ) is replaced by  and , if each occurrence of a pre-declared function (like sin, cos, exp, etc.) is replaced by the corresponding pre-declared interval function, and if the arithmetic operations in fmag(, ) are replaced by the corresponding interval arithmetic operations, is called as the natural interval extension of  fmag(,) to  and . The natural interval extension of fmag is denoted as Fmag (, ). Similarly for Fang (, ) and F (, ).

Interval frequency plant template. By extension of the definition for the point frequency case, the plant template at frequency interval  can be defined as G() ={f(,),}.

The plant template for the case of frequency interval can also be generated using any of the IATG algorithms in section 2, by considering  as just another interval parameter for purposes of subdivision. The resulting template is called as an interval frequency plant template. An interval frequency plant template also comprises of one or more phase-magnitude rectangles, called as template rectangles, just as for the point frequency case. A key property of the interval frequency plant template at  is that it always encloses the exact template G(). That is, the interval frequency plant template at  contains all the points in the usual (point frequency) templates generated at all .

Working phase interval. The working phase interval Y is now a function of frequency, as zs depends on frequency through the spec. The working phase interval Y corresponding to the  under consideration can be found from (4).

Spacing the frequency intervals and the bounds. The designer can choose among various criteria for obtaining frequency intervals from the given design frequency range. The following criterion is chosen here. Let Ω' be an adjacent frequency interval to . Then, the maximum of the upper bound at  and maximum of the upper bound at Ω'should be within some specified distance or spacing, denoted  dB (note that the maximums of the bounds need not occur at the same ). This criterion can be written as (with all quantities in dB)

(7)    minFmag(Ω,Λ)−minFmag(Ω',Λ)≤Τ−β

where Fmag is the natural interval extension of the plant magnitude function (cf. section 6.1), and  is the overestimation in Fmag.  Similarly for the spacing between the minimums of the lower bounds at  and. Ω'.

The above criterion is chosen just to illustrate the methodology of the proposed procedure. Other criteria can be specified by the designer to suit the needs of the problem, and the proposed procedure can be suitably adapted to achieve the ends.

6.1 The procedure

A unified procedure for obtaining gain-phase margin bounds is outlined below. The procedure generates bounds that are of prescribed accuracy  dB, and spaced such that bounds of adjacent frequency intervals have their maximum magnitudes within  dB, and similarly for the minimum magnitudes.

The steps of the unified procedure for robust gain-phase margin specification are

- Split the accuracy: The accuracy  is split into two positive quantities Q and G such that their sum equals .

- Subdivide the design frequency range:  The design frequency range is successively bisected till (7) is achieved at each frequency interval .

- Subdivide the working phase intervals: At each frequency interval , the following is done. The working phase interval Y in (4) is first computed. Then, Y is successively bisected till over each subdivision X, the width (in dB) of function Q  in (2) is at most Q. Lastly, the smallest width of these subdivisions is found and denoted as w.

- IATG at each frequency interval: For each frequency interval , using any IATG algorithm an interval frequency plant template comprising of template rectangles of prescribed size is generated: each template rectangle should have its magnitude side of length at most Q dB, and phase side of length at most w. Then, the boundary template rectangles are extracted, while the rest are discarded from each interval frequency plant template.

- IABG at each frequency interval: At each frequency interval , using the X intervals obtained in step 3 above, the bound intervals are composed as in section  3.1.3 and then the bounds are composed from these bound intervals as in section 3.1.4.

6.2. Illustrative Example for class C problem

Example 6.1. The transfer function of a system found in active noise and vibration control is given as Example 1 in the Appendix. The system has highly under-damped resonance. The nominal parameters are n0=0.75 and 0=0.02. The robust gain-phase margin specification is wm=1.2 dB. The design frequency range is 0 =[ 0.5,1.5]. The objective is to generate the bounds with an accuracy =2.5 dB and spacing =7.5 dB (cf. equation 7) over the given design frequency range.

Notes: The design frequencies (as points or as intervals) and the controller phases at which bounds are to be generated are not selected here, but are rather left to be appropriately determined by the procedure. However, the design frequency range, the accuracy of the bounds, and the spacing between them are prescribed. This puts the design problem in class C.

The above unified procedure is applied to generate the robust stability bounds of prescribed accuracy and spacing. In Step 1, the prescribed accuracy =2.5 dB is split as Q =2.1, G =0.4 dB, so that =Q + G. In step 2, the given design frequency interval 0 = [0.5,1.5] is successively bisected till the spacing inequality in (7) is achieved at each frequency interval . This step gave 17 frequency intervals listed in Table 2. Then, steps 3 and 4 are executed at each frequency interval to generate the interval frequency template and extract the boundary rectangles, as detailed in the earlier example. Lastly, the IABG algorithm for the robust gain-phase margin specification, Algorithm RSBI in Nataraj and Sardar ( 2000a), is applied to the boundary rectangles of the templates to generate the bounds at each frequency interval. The total time for these steps is about 300 seconds. For plotting purposes, the controller bounds are converted to those on the nominal loop transmission function Lo (s). Figure 3 shows the plots of these bounds at selected frequency intervals 4,  5 and ,  6  (bounds only for selected frequencies are plotted, to avoid cluttering the figure).

Table 2.  Design frequency intervals obtained by subdividing the given design frequency range in Example 6.1

1=[0.50,0.56] 2 =[0.56,0.62] 3=[0.62,0.65] 4 =[0.65,0.68] 5=[0.68,0.75] 6 =[0.75,0.78] 7=[0.78,0.81] 8=[0.81,0.84] 9=[0.84,0.87] 10=[0.87,0.90] 11=[0.90,0.93] 12=[0.93,1.00] 13=[1.00,1.06] 14=[1.06,1.12] 15=[1.12,1.25] 16=[1.24,1.37] 17=[1.37,1.50]

Figure 3. Robust gain-phase margin bounds for the under-damped system in Example 6.1. The bounds are generated by the unified procedure for class C problems. Seventeen design frequency intervals are automatically generated, but bounds for only 3 frequency intervals are plotted and marked as A - 4,  B - 5,  C - 6. The specified bound accuracy is 2.5 dB.

Comments on the automatic selection of frequency intervals: It is seen in Figure 3 that the frequency intervals are such that the spacing between the bounds (in the sense of section 6.4) is at most =7.5 dB. Further, it is seen in Table 2 that the frequency intervals have different widths, with the maximum width as 0.13 and the minimum width as 0.03. The widths vary because the frequency intervals are automatically found in the unified procedure (in the subdivision Step 2), so that bounds of prescribed accuracy are generated.

In contrast, in the point QFT procedures, the designer (usually arbitrarily) selects the frequency values at which the design is to be performed. The unified procedure completely solves the difficulty of carefully analyzing and selecting design frequencies with the point QFT procedures. To emphasize this capability, note in Figure 3 that while proceeding from 4 through 6, the bounds shift considerably rightward at 5. This phenomenon is important for loop-shaping, and the unified procedure captures it automatically. However, it is quite likely that this phenomenon would be missed in a designer-selected frequency set.

Comments on the accuracy of the bounds: The maximum possible error in the generated bounds is readily obtainable from the widths of the plotted intervals in Figure 3. It is seen from the figure that this maximum error is less than the prescribed accuracy of =2.5 dB.

Comments on the automatic selection of phase intervals: The phase intervals plotted in Figure 3 have different widths at a given frequency interval. Further, the widths also vary with the frequency interval. The widths vary in this manner because they are adaptively and automatically found in the unified procedure (in the subdivision step 3), so that bounds of prescribed accuracy can be generated. In contrast, in the point QFT procedures, the designer arbitrarily selects the phase values at which the bounds are to be generated.

7. CONCLUDING REMARKS

It has been shown in this paper how IQFT lends rigor and reliability to Horowitz's basic QFT design technique. Several fundamental issues that have been so far treated in a rather ad-hoc manner in QFT, such as selection of design frequencies, phases, and plant parameter combinations, are treated in a systematic fashion in IQFT.

Much, however, remains to be added to the baggage of IQFT tools, even for the basic single input-output linear plant case. Preliminary work done by Vamsikrishna (2000) on loop-shaping using global optimization tools of IA has met with some success, but needs to be made computationally more efficient and tested on more examples. The computational verification of existence of a solution to the basic QFT design problem seems to be possible, and is an exciting prospect in IQFT. Further, the cases of multivariable and nonlinear plants are as yet untouched in IQFT.

REFERENCES

F. N. Bailey, D. Panzer, and G. Gu. (1988). Two          algorithms for frequency domain design of          control systems. International Journal of           Control, 48, 1787-1806.

C. Borghesani, Y. Chait and O. Yaniv. (1995). The         quantitative feedback theory toolbox for Matlab,     The MathWorks Inc. , MA, USA.

Y. Chait, C. Borghesani and Y. Zheng. (1995).         Single loop QFT design for robust  performance in the presence of nonparametric uncertainties. Trans. of the ASME Journal of Dynamic Systems, Measurement and Control,  117, 420-424.

Y. Chait and C. V. Hollot. (1990). A comparison        between H-infinity methods and QFT for a SISO plant with both parametric uncertainty and performance specifications. In O.D.I. Nwokah, editor, Recent developments in quantitative feedback theory, 33-40.

Y. Chait and O. Yaniv. (1993). Multi-input /single-       output computer-aided control design using       quantitative feedback theory. International       Journal of Robust and Nonlinear Control,  3, 47-54.

W. Chen and D.J. Balance. (1999). Plant Template        Generation for Uncertain Plants in QFT. Trans. of the ASME Journal of DynamicSystems, Measurement and Control, 121, 359-364.

B. Cohen, M. Nordin and P.O. Gutman. (1995).       Recursive grid methods to compute value sets of transfer functions with parametric uncertainty. In Proc. Of ACC, 3861-3865.

I. M. Horowitz. (1991). Survey of quantitative feedback        theory (QFT). International Journal of       Control, 53, 255-291.

I. M. Horowitz. (1993). Quantitative feedback design        theory (QFT). QFT Publications, Boulder, Colorado, USA.

S. Jayasuriya. (1993). Frequency domain design for       robust performance under parametric,       unstructured, or mixed uncertainties. Trans. of the      ASME journal of Dynamic Systems, Measurement       and  Control, 115,  439-451.

R. B. Kearfott. (1987). Abstract generalized bisection        and   cost bound. Mathematics of computation, 49,   187-202.

R. B. Kearfott. (1987).  Some tests of generalized         bisection. ACM Transactions on Mathematical        Software, 13, 197-220.

R. Klatte, U.Kullisch, M. Neaga et.al. (1993).        PASCAL-XSC language reference with examples. Springer-Verlag, Berlin.

R. E. Moore. (1979). Methods and applications of        interval analysis. SIAM, Philadelphia.

R. E. Moore. (1991). Global optimization to        prescribed accuracy. Computers Math. Appl., 21, 25-39.

P. S. V. Nataraj (2001). Computation of QFT bounds for robust tracking specifications. Automatica, accepted.

P. S. V. Nataraj and A. K. Prakash. (2001).  On     subdivision direction selection for QFT template generation. In preparation.

P.S.V.Nataraj and G.Sardar. (2000a). Computation of QFT bounds for robust  sensitivity and gain-phase margin. Trans. of the ASME    Journal of Dynamic Systems, Measurement  and Control,  36, 111-119.

P. S. V. Nataraj and G. Sardar. (2000b).  Template         generation for continuous transfer functions        using interval analysis, Automatica, 122, 528-534.

P. S. V. Nataraj and S. Sheela. (2001a). An algorithm for extraction of boundary rectangles from interval        QFT templates. Submitted.

P. S. V. Nataraj and S. Sheela. (2001b). A template generation  algorithm using parallel function        evaluations and adaptive subdivisions. Submitted.

P. S. V. Nataraj and S. Sheela. (2001c). A Unified procedure for generation of QFT bounds to prescribed accuracy.  Submitted.

O.D.I.Nwokah, S.Jayasuriya and Y.Chait. (1992).  Parametric robust control by quantitative feedback theory. AIAA Journal of Guidance and Control, 5, 207-214.

L. B. Rall.  (1981). Automatic differentiation,         techniques and applications. Lecture Notes in Computer Science, 120, Springer-Verlag, Berlin.

D. Ratz and T. Csendes. (1995). On the selection of         subdivision directions in interval branch-and-        bound methods for global optimization. Journal of Global Optimization, 7, 183-207.

J. M. Rodrigues, Y.Chait and C.V. Hollot. (1997).         An efficient algorithm for computing QFT         bounds. Trans. of the ASME Journal of Dynamic Systems, Measurement and Control, 119, 548-552.

S. M. Rump. (1999). INTLAB - Interval laboratory. In T. Csendes, Editor, Developments in reliable computing. Kluwer Academic  Publishers.

G. Sardar and P. S. V. Nataraj. (1997). A template generation  algorithm for non-rational transfer        functions in QFT designs. In Proc. 36th  IEEE        Conf. Decision and Control,  2684-2689, San        Diego, USA.

S. Sheela and P. S. V. Nataraj. (2001a). A new subdivision strategy for range computations. Reliable Computing, to appear.

S. Sheela and P. S. V. Nataraj. (2001b). A reliable   parallel - adaptive algorithm with back-tracking       for QFT template generation. IETE Jl. Technical Review. To appear.

M. Sidi. (1976). Feedback synthesis with plant        ignorance, non-minimum phase and time domain tolerances. Automatica, 12, 265-271.

D. F. Thompson and O.D.I.Nwokah. (1994).        Analytical loop shaping methods in quantitative feedback theory. Trans. of the ASME Journal of Dynamic Systems, Measurement and Control, 116,  169-177.

P. Vamsikrishna. (2000). Design of controllers        using global optimization techniques. M. Tech thesis, Systems and Control Engg. Group, IIT Bombay, India.

O. Yaniv and I. Horowitz. (1987). Quantitative       feedback theory- reply to criticisms. International Journal of Control, 40,  945-962.

Y. Zhao and S. Jayasuriya. (1998). An H-infinity        formulation of quantitative feedback theory.        Trans. of the ASME Journal of Dynamic Systems, Measurement and Control, 120,  305-313.

APPENDIX A. LIST OF EXAMPLES

The following is a suite of transfer function examples used in this work. The test problems are taken from the QFT literature. The problem names reflect the general type of the system.

Example 1. Under-damped Second Order System (Rodrigues et. al., 1997): The transfer function for a system occurring in active noise and vibration control with highly underdamped resonances is

g(s)=ωn2s2+2ξωns+ωn2ωn∈0.75,1.25,ξ∈0.02,0.06,ω=1.

Example 2. DC Motor (Bailey et.al.,1988): The DC Motor drives a viscously damped inertial load. The transfer function between the torque and armature voltage is

g(s)=K(Jls+Bl)(Ls+R)(Jms+Jls+Bm+Bl)+K2K∈[0.2,0.6],Jl∈[10−5,3∗10−5],Jm=2∗10−3Bm=2∗10−5,L=10−2H,R=1Ω,Bl=Bm,ω=20.

Example 3. Simple poles (Nataraj and Sardar, 2000a): The transfer function for a stable second order system with real poles is

g(s)=k(s+a)(s+b)a∈1,5,b∈20,30,k∈1,10,ω=1.

Example 4. Non-minimum phase (Sidi, 1976): The transfer function for a non-minimum phase system with real poles and zeros is

g(s)=K(1−Ds)s(1+Bs),B∈0.3,1,D∈0.05,0.1,K∈1,3,ω=1.

Example 5. Non-rational (Horowitz, 1993, pp.129):  The transfer function for a non-rational system is

g(s)=e−sT1+be−as,a∈1,2b∈0.4,0.6,T∈0.01,0.02,ω=2.

Example 6. Electro-Mechanical (Cohen et. al., 1995): The transfer function between control torque to motor speed of an electro-mechanical system is

g(s)=Jls2+ds+kJlJms3+(Jl+Jm)ds2+(Jl+Jm)ksJm=0.4,Jl∈[5.6,8],d∈[30,300],k∈5880,5900,ω=10π.

Example 7. Vehicle clutch system (Chen and Balance, 1999): The transfer function between the input clutch position to the output transmission speed is

)

(

)

(

2

2

2

2











































c

r

s

v

s

c

r

s

v

s

e

v

s

v

s

c

j

g

k

j

k

s

j

g

c

j

c

s

s

j

j

k

j

c

s

k

s

g

jv = [1400,11000], ks = [5800,115000], kc = [100,800], jc = 0.09, je = 3.07, cs = 377, gr = 27.0, =10.

Example 8. Multiple transport lags (Nataraj and Sardar, 2000b): The transfer function for a system with multiple transport lags is

g(s)=T+τ1e−sT1log10T1−e−sT(1+cosτ1s)+1−e−sTrsTr1+sTr4π2T1∈3,5,Tr∈0.5,0.7,T∈9.25,9.35,τ1∈0.49,0.5,ω=0.5.

Example 9. Mechanical system ((Horowitz, 1993, pp. 222 ) : The transfer function for a mechanical system is

g(s)=kms(s2fm2+bms+c),f∈[1,2]m2∈[1,10],b∈[0.5,1],c∈[2,3]k∈[0.5,2],ω=8.

Example10. Aircraft,longitudinalMotion  (Thomspon and Nwokah, 1994):  The transfer function for the longitudinal motion of an aircraft is (from aerodynamic data, the ranges for the uncertain parameters are identified)

g(s)=k(1+sz)s(1+sp)(1+2ξωns+s2ωn2),z∈[0.5,0.75],p∈[1,10],ξ∈[0.8,0.9]ωn∈[5,6],k∈[0.2,2],ω=0.1.

Example 11. Inverted pendulum (Borghesani et. al, 1995): The transfer function between pendulum angle to the cart's motor current is

g(s)=Kαωn2(1L)s2e−sτs(s+α)−KgfKαs2+2ξωns+ωn2L∈[0.3,0.45],K∈[1.5,1.7],ξ∈[0.001,0.02],τ∈[0.014,0.015],α∈[15,17],ωn∈[50,70],Kgf∈[0.005,0.15],g=9.81,ω=10.